Section 724.410  Closure and Post-Closure Care

a)	At final closure of the landfill or upon closure of any cell, the owner or operator must cover the landfill or cell with a final cover designed and constructed to do the following:

1)	Provide long-term minimization of migration of liquids through the closed landfill;

2)	Function with minimum maintenance;

3)	Promote drainage and minimize erosion or abrasion of the cover;

4)	Accommodate settling and subsidence so that the cover's integrity is maintained; and

5)	Have a permeability less than or equal to the permeability of any bottom liner system or natural subsoils present.

b)	After final closure, the owner or operator must comply with all post-closure requirements contained in Sections 724.217 through 724.220, including maintenance and monitoring throughout the post-closure care period (specified in the permit under Section 724.217).  The owner or operator must do the following:

1)	Maintain the integrity and effectiveness of the final cover, including making repairs to the cap as necessary to correct the effects of settling, subsidence, erosion, or other events;

2)	Continue to operate the leachate collection and removal system until leachate is no longer detected;

3)	Maintain and monitor the LDS in accordance with Sections 724.401(c)(3)(D) and (c)(4) and 724.403(c), and comply with all other applicable LDS requirements of this Part;

4)	Maintain and monitor the groundwater monitoring system and comply with all other applicable requirements of Subpart F;

5)	Prevent run-on and run-off from eroding or otherwise damaging the final cover; and

6)	Protect and maintain surveyed benchmarks used in complying with Section 724.409.

(Source:  Amended at 42 Ill. Reg. 22614, effective November 19, 2018)