Section 251.40  Tagging

a)	A certification tag shall be affixed to the portable fire extinguisher or fire suppression system when the extinguisher or system is installed or modified, or when maintenance is performed.

b)	All certification tags that are exposed to the outdoor elements shall be enclosed in a tag protector, unless the certification tag is made of a weather resistant material.

c)	On pre-engineered or engineered fire suppression systems, the certification tag shall be attached and visible at the suppression unit and at the point of manual activation or inspection.

d)	Tagging of fire equipment shall be performed only by a licensed fire equipment employee.

e)	Certification tags shall conform to all specifications and requirements listed in Section 251.50.

(Source:  Amended at 47 Ill. Reg. 16058, effective October 26, 2023)